RISK ASSESSMENT

Dental Amalgam & Mercury Handling

Reference: BS-RA-DENT-004

Assessment Date: [Insert Date]

Assessor: [Insert Name]

Practice: [Insert Practice Name & Address]

Review Frequency: Annual (or following any incident/significant change)

1. SCOPE & PURPOSE

This risk assessment covers the safe handling, placement, removal, storage, and disposal of dental amalgam and mercury within the dental practice, including environmental compliance with mercury waste regulations.

2. RELEVANT LEGISLATION & GUIDANCE

- Health and Safety at Work etc. Act 1974 (HSWA) – Sections 2 & 3

- Control of Substances Hazardous to Health Regulations 2002 (COSHH) (SI 2002/2677)

- Workplace Exposure Limits (EH40, 4th Ed., HSE) – Mercury WEL: 0.02 mg/m³ TWA

- EU Regulation 2017/852 on Mercury (retained EU law) – restrictions on amalgam use

- Environmental Permitting (England and Wales) Regulations 2016

- Hazardous Waste (England and Wales) Regulations 2005 (SI 2005/894)

- Water Industry Act 1991 – discharge of mercury to foul sewer

- The Dentists Act 1984 (as amended) – scope of dental practice

- BDA Advice Sheet: Amalgam and Mercury – Safety and Environmental Guidance

- CQC Fundamental Standards – Regulation 12 & Regulation 15

3. RISK RATING METHODOLOGY

Risk Rating = Likelihood (L) × Severity (S). Likelihood: 1 (Rare) to 5 (Almost Certain). Severity: 1 (Negligible) to 5 (Catastrophic).

| 1–3 (LOW) | Acceptable risk, monitor | 8–12 (MEDIUM) | Reduce risk where practicable |
| --- | --- | --- | --- |
| 4–6 (LOW-MED) | Monitor, review controls | 15–25 (HIGH) | Immediate action required |

4. RISK ASSESSMENT TABLE

| Hazard | Risk | Who at Risk | L | S | Rating | Control Measures | L | S | Residual |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mercury vapour inhalation (placement/removal) | Chronic mercury poisoning: tremor, renal damage, neuropsychiatric symptoms | Staff/Patient | 3 | 4 | 12 (MEDIUM) | COSHH assessment for mercury; high-volume aspiration during amalgam removal; rubber dam isolation; water coolant spray; surgery ventilation (minimum 6 air changes/hour); pre-capsulated amalgam only (no loose mercury); mercury vapour monitoring | 1 | 3 | 3 (LOW) |
| Mercury spillage | Environmental contamination, acute vapour exposure | Staff | 2 | 4 | 8 (MEDIUM) | Mercury spill kit in designated location (sulphur powder, sealed container, PPE); spill protocol displayed; contaminated area ventilated and sealed; no vacuuming of mercury; incident reported and recorded; annual spill drill | 1 | 3 | 3 (LOW) |
| Patient exposure (amalgam placement in restricted groups) | Mercury exposure to vulnerable patients | Patient | 2 | 3 | 6 (LOW-MED) | EU Regulation 2017/852 compliance: amalgam not used in children under 15, pregnant/breastfeeding women (unless deemed strictly necessary by practitioner); alternatives offered and documented; informed consent | 1 | 2 | 2 (LOW) |
| Amalgam waste entering waterways | Environmental mercury contamination, regulatory prosecution | Public/Environment | 3 | 4 | 12 (MEDIUM) | ISO 11143-compliant amalgam separator installed on all dental chair suction lines; separator serviced per manufacturer schedule; discharge monitored; spent amalgam collected by licensed hazardous waste contractor; consignment notes retained | 1 | 3 | 3 (LOW) |
| Improper amalgam waste storage | Mercury vapour release, fire risk, prosecution | Staff | 2 | 3 | 6 (LOW-MED) | Waste amalgam (contact and non-contact) stored in sealed containers with photographic fixer or glycerine; containers labelled per hazardous waste regulations; stored in ventilated area; collected by licensed contractor at regular intervals | 1 | 2 | 2 (LOW) |
| Staff chronic low-level exposure | Cumulative mercury absorption, occupational disease | Staff | 3 | 3 | 9 (MEDIUM) | Annual workplace mercury vapour monitoring (must remain below 0.02 mg/m³ TWA per EH40); health surveillance for exposed staff; urine mercury testing available on request; no carpet in surgeries (mercury traps); pre-capsulated amalgam exclusively | 1 | 2 | 2 (LOW) |

5. ADDITIONAL CONTROL MEASURES & EMERGENCY PROCEDURES

- COSHH assessment for mercury and amalgam reviewed annually; safety data sheets accessible to all clinical staff.

- Amalgam separator maintenance log: service dates, contractor details, and certificates of proper disposal retained for minimum 3 years.

- Mercury spill kit contents checked quarterly: sulphur powder, disposable scoop, sealable polythene bag, warning sign, PPE (nitrile gloves, goggles).

- Staff training on mercury handling, spill response, and environmental obligations completed at induction and updated annually.

- Environmental permit conditions reviewed annually; compliance confirmed with local water authority regarding discharge of trade effluent.

6. MONITORING & REVIEW

This risk assessment shall be reviewed annually, or sooner if there is a significant change in working practices, an incident occurs, or new legislation/guidance is published. All staff must be made aware of the contents of this document and sign to confirm understanding.

7. SIGN-OFF

I confirm that this risk assessment has been carried out in accordance with relevant legislation and that the control measures identified are suitable and sufficient.

| Assessed By: [Insert Name] | Position: [Insert Name] | Date: [Insert Name] | Review Date: [Insert Name] |
| --- | --- | --- | --- |

© Barker Scott. All rights reserved. This document is confidential and intended for the sole use of the named dental practice.